National Textile University, Faisalabad

Department of Computer Science

| Name: | Hiba Fatima |
| --- | --- |
| Class: | BSCS_A 5th Semester |
| Registration No: | 23-NTU-CS-1037 |
| Assignment : | Project |
| Course Name: | Embedded IOT |
| Submitted To: | Sir Nasir |
| Submission Date: | 26-10-2025 |

Project Documentation: Multi-Task ESP32 Control System

1. Project Overview

This project implements two distinct tasks—LED mode cycling (Task A) and press-type detection (Task B)—on a single ESP32 Dev Module. The system is controlled by two pushbuttons and provides visual feedback via an OLED display and audible feedback via a buzzer.

| Component | Quantity | Purpose |
| --- | --- | --- |
| ESP32 Dev Module | 1 | Microcontroller running the logic. |
| OLED Display (SSD1306, I²C) | 1 | Displays the current mode and button event. |
| Push Button | 2 | Inputs: one for mode cycle/reset, one for press detection. |
| LED (Red, Green, Blue) | 3 | Visual outputs for mode sequences and toggling. |
| Buzzer | 1 | Audible output for the long-press event. |

TASK A:

Circuit Diagram: Design a Wokwi circuit and draw a neat hand-sketch including:

• 2 push buttons

• 3 LEDs

• 1 buzzer

• 1 OLED

Task A — Coding: Use one button to cycle through LED modes (display the current state on

the OLED):

1. Both OFF

2. Alternate blink

3. Both ON

4. PWM fade

Use the second button to reset to OFF.

TASK A  :FIXED BUTTON ISSUE

WOKWI LINK  :

https://wokwi.com/projects/445873016530129921

SCREENSHOT:

Both OFF:

Alternate blink:

Both ON:

PWM fade:

Loom Link Video Link:

https://www.loom.com/share/f5a0f906af794c0ab559880db1026b48

Code Of The Program:

//Hiba Fatima

//23-Ntu-cs-1037

//Task A — Coding: Use one button to cycle through LED modes (display the current state on

//the OLED):

//1. Both OFF

//2. Alternate blink

//3. Both ON

//4. PWM fade

//Use the second button to reset to OFF.

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// -------- OLED CONFIG ----------

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// -------- PIN DEFINITIONS ----------

#define LED1 25        // Red

#define LED2 26        // Yellow

#define LED3 27        // Optional Green

#define BUTTON_MODE 32 // Mode button

#define BUTTON_RESET 33 // Reset button

#define BUZZER 14

// -------- GLOBAL VARIABLES ----------

int mode = 0; // 0: OFF, 1: Alternate Blink, 2: Both ON, 3: PWM Fade

unsigned long lastPress = 0;

int fadeValue = 0;

int fadeDir = 1;

// -------- FUNCTION DECLARATIONS ----------

void showMode();

void setup() {

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(LED3, OUTPUT);

pinMode(BUZZER, OUTPUT);

pinMode(BUTTON_MODE, INPUT_PULLUP);

pinMode(BUTTON_RESET, INPUT_PULLUP);

Wire.begin();

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(0, 0);

display.println("System Ready...");

display.display();

delay(1000);

showMode();

}

void loop() {

// ----- MODE BUTTON -----

if (digitalRead(BUTTON_MODE) == LOW) {

delay(200); // debounce

mode = (mode + 1) % 4;

showMode();

while (digitalRead(BUTTON_MODE) == LOW); // wait for release

}

// ----- RESET BUTTON -----

if (digitalRead(BUTTON_RESET) == LOW) {

delay(200);

mode = 0;

showMode();

while (digitalRead(BUTTON_RESET) == LOW);

}

// ----- LED BEHAVIOR -----

switch (mode) {

case 0: // OFF

digitalWrite(LED1, LOW);

digitalWrite(LED2, LOW);

break;

case 1: // Alternate blink

digitalWrite(LED1, HIGH);

digitalWrite(LED2, LOW);

delay(300);

digitalWrite(LED1, LOW);

digitalWrite(LED2, HIGH);

delay(300);

break;

case 2: // Both ON

digitalWrite(LED1, HIGH);

digitalWrite(LED2, HIGH);

break;

case 3: // PWM fade on LED1

analogWrite(LED1, fadeValue);

analogWrite(LED2, fadeValue);

fadeValue += fadeDir * 10;

if (fadeValue >= 255 || fadeValue <= 0) fadeDir = -fadeDir;

delay(30);

break;

}

}

// -------- DISPLAY FUNCTION ----------

void showMode() {

display.clearDisplay();

display.setCursor(0, 0);

display.println("LED Mode System");

display.setCursor(0, 20);

switch (mode) {

case 0: display.println("Mode: OFF"); break;

case 1: display.println("Mode: Alternate Blink"); break;

case 2: display.println("Mode: Both ON"); break;

case 3: display.println("Mode: PWM Fade"); break;

}

display.display();

}

HANDMADE SKETCH:

TASK B:

— Coding: Use a single button with press-type detection (display the event on the OLED): • Short press → toggle LED • Long press (> 1.5 s) → play a buzzer tone.

WOKWI LINK:

https://wokwi.com/projects/445873593457762305

WORKING SCREENSHOT:

Buzzer running:

Loom video link:

https://www.loom.com/share/28a3c60e49ba4aefb3ebd774c85f3a5d

Code of the program:

//Hiba Fatima

//23-Ntu-cs-1037

//Task B — Coding: Use a single button with press-type detection (display the event on the

//OLED):

// Short press → toggle LED

//Long press (> 1.5 s) → play a buzzer tone

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// ---------- OLED CONFIG ----------

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// ---------- PIN DEFINITIONS ----------

#define BUTTON 32

#define LED 25

#define BUZZER 14

// ---------- GLOBAL VARIABLES ----------

bool ledState = false;

unsigned long pressStart = 0;

bool isPressed = false;

// ---------- SETUP ----------

void setup() {

pinMode(BUTTON, INPUT_PULLUP);   // button connected to GND

pinMode(LED, OUTPUT);

pinMode(BUZZER, OUTPUT);

Wire.begin();

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(1);

display.setCursor(0, 0);

display.println("System Ready...");

display.display();

delay(1000);

showMessage("Waiting for button...");

}

// ---------- MAIN LOOP ----------

void loop() {

int buttonState = digitalRead(BUTTON);

// detect press

if (buttonState == LOW && !isPressed) {

isPressed = true;

pressStart = millis(); // time press began

}

// detect release

if (buttonState == HIGH && isPressed) {

isPressed = false;

unsigned long pressDuration = millis() - pressStart;

if (pressDuration > 1500) {

// Long press -> play buzzer melody

playMelody();

showMessage("Long Press -> Melody");

} else {

// Short press -> toggle LED + short beep

ledState = !ledState;

digitalWrite(LED, ledState);

shortBeep();

showMessage("Short Press -> LED " + String(ledState ? "ON" : "OFF"));

}

}

}

// ---------- FUNCTIONS ----------

void playMelody() {

// simple 3-tone melody for long press

tone(BUZZER, 500, 200);   // low tone

delay(250);

tone(BUZZER, 800, 200);   // medium tone

delay(250);

tone(BUZZER, 1200, 300);  // high tone

delay(350);

noTone(BUZZER);

}

void shortBeep() {

// quick beep for short press

tone(BUZZER, 1000, 100);

delay(150);

noTone(BUZZER);

}

void showMessage(String msg) {

display.clearDisplay();

display.setCursor(0, 0);

display.println("Button Event:");

display.setCursor(0, 20);

display.println(msg);

display.display();

}

Handmade sketch:

GITHUB LINK:

https://github.com/HIBA-616/Embedded-iot-1037-.git